BOD INPUT/OUTPUT Connection.

| INPUT CONNECTION |  |  |  |
| --- | --- | --- | --- |
| SR.No. | Identification Ferule | Supply | Description. |
| 01 | +24V | +24V |  |
| 02 | -V | -V |  |
| 03 | E | E |  |
| 04 | -V |  | Stand By Compressor Feed back |
| 05 | +24V |  |  |
| 06 | I1 | +24V | Dry Heater Feed back |
| 07 | I2 | +24V | Compressor No. 01 Feed back |
| 08 | I3 | +24V | Compressor No. 02 Feed back |
| 09 | I4 | +24V | Mains Feed back |
| 10 | I5 | +24V | Door Switch Feed back |
| 11 | -V |  |  |
| 12 | 1 |  | Temperature Sensor no. 01 |
| 13 | 2 |  | Temperature Sensor no. 02 |
| OUTPUT CONNECTION |  |  |  |
| 01 | +24V | +24V |  |
| 02 | -V | -V |  |
| 03 | PAH | +24V | PLC Alarm Hooter |
| 04 | 01 | +24V | Dry Heater SSR |
| 05 | PDHS | +24V | PLC Dry Heater Safety |
| 06 | PCO1 | +24V | PLC Compressor No. 01 |
| 07 | PCO2 | +24V | PLC Compressor No. 02 |

Transit Relay Card.

| 01 | -V | -V |  | All Wires Coming from PLC Output. (Starting with P) |
| --- | --- | --- | --- | --- |
| 02 | PCO2 | +24V | PLC COMPRESSOR 02 |  |
| 03 | -V | -V |  |  |
| 04 | PCO1 | +24V | PLC COMPRESSOR 01 |  |
| 05 | -V | -V |  |  |
| 06 | PDHS | +24V | Dry Heater Safety |  |
| 07 | -V | -V |  |  |
| 08 | PAH | +24V | PLC ALARM HOOTER |  |

| 01 | L4 | 230 V AC |  | All Wires Going to Relay/contactor/terminals. Transit Relay card is only 5Amps.DO not connect heavy components directly on the Card. |
| --- | --- | --- | --- | --- |
| 02 | 09 | 230 V AC | Compressor 02 Output 230 V AC |  |
| 03 | L4 | 230 V AC |  |  |
| 04 | 08 | 230 V AC | Compressor 02 Output 230 V AC |  |
| 05 | L2 | 230 V AC |  |  |
| 06 | 07 | 230 V AC | Dry Heater Safety |  |
| 07 | +24V | +24V |  |  |
| 08 | 00 | +24V | Alarm Hooter Output +24V |  |